Program:

clc;

clear all;

%----------Input Section---------------

Hbts = 40; %Height measured from the base of the BTS tower to the radiation centerline

Tbts = 350; %Terrain elevation at the location of the BTS

Htav = 300; %Height of the average terrain (from 3 Km to 15 km distance from the BTS)

Hm = 2; %Height of the mobile antenna in meters

f = 2100; %100:100:3000; %Range of frequencies in MHz

d = 0.5:0.5:15; %Range of Tx-Rx separation distances in Kilometers

Pt = 0.020; %Power transmitted by the BTS antenna in Watts

Gt = 10; %BTS antenna gain in dBi

%--------------------------------------

Hb = Hbts + Tbts - Htav; %Effective Height of the BTS antenna in meters

C = 0;

aHm = 3.2 * (log10(11.75 * Hm)) ^ 2 - 4.97;

A = 69.55 + 26.16 * log10(f) - 13.82 * log10(Hb) - aHm;

B = 44.9 - 6.55 * log10(Hb);

PL = A + B * log10(d) + C;

subplot(2, 1, 1)

plot(d, PL, 'r', 'LineWidth', 2);

title('Hata-Okumura Path Loss Model', 'Fontsize',20);

xlabel('Distance - Kilometers', 'Fontsize',16);

ylabel('Path Loss (dB)', 'Fontsize',16);

%Compute Received Signal Level

Pr = 10 * log10(Pt * 1000) + Gt - PL

subplot(2, 1, 2)

plot(d, Pr, 'r', 'LineWidth', 2);

title('Hata-Okumura Model', 'Fontsize',20);

xlabel('Distance - Kilometers', 'Fontsize',16);

ylabel('Received Signal Level (dBm)', 'Fontsize',16);

Output: