Program:

% Simulation of OFDM system in an AWGN environment

% Follows IEEE 802.11 specification

clear; clc;

% -------- Simulation parameters ----------------

nSym = 10^4; % Number of OFDM Symbols to transmit

EbN0dB = -20:2:8; % Bit to noise ratio

% -------- OFDM Parameters - Given in IEEE Spec --

N = 64; % FFT size or total number of subcarriers (used + unused) 64

Nsd = 48; % Number of data subcarriers 48

Nsp = 4;  % Number of pilot subcarriers 4

ofdmBW = 20 * 10^6; % OFDM bandwidth

% -------- Derived Parameters --------------------

deltaF = ofdmBW / N; % 20 MHz / 64 = 0.3125 MHz

Tfft = 1 / deltaF; % IFFT/FFT period = 3.2 µs

Tgi = Tfft / 4; % Guard interval duration - duration of cyclic prefix

Tsignal = Tgi + Tfft; % Duration of BPSK-OFDM symbol

Ncp = N * Tgi / Tfft; % Number of symbols allocated to cyclic prefix

Nst = Nsd + Nsp; % Number of total used subcarriers

nBitsPerSym = Nst; % For BPSK, the number of Bits per Symbol equals the number of subcarriers

% Convert to symbol to noise ratio

EsN0dB = EbN0dB + 10 * log10(Nst / N) + 10 * log10(N / (Ncp + N));

% Initialize error counters

errors = zeros(1, length(EsN0dB));

theoreticalBER = zeros(1, length(EsN0dB));

% Monte Carlo Simulation

for i = 1:length(EsN0dB)

for j = 1:nSym

% ----------------- Transmitter --------------------

s = 2 * round(rand(1, Nst)) - 1; % Generating Random Data with BPSK modulation

% IFFT block

% Assigning subcarriers from 1 to 26 (mapped to 1-26 of IFFT input)

% and -26 to -1 (mapped to  38 to 63 of IFFT input); Nulls from 27 to 37

% and at 0 position

X_Freq = [zeros(1,1), s(1:Nst/2), zeros(1,11), s(Nst/2+1:end)];

% Converting from frequency domain to time domain

x_Time = N / sqrt(Nst) * ifft(X_Freq);

% Adding Cyclic Prefix

ofdm_signal = [x_Time(N-Ncp+1:N), x_Time];

% -------------- Channel Modeling ----------------

noise = 1/sqrt(2) * (randn(1, length(ofdm_signal)) + 1i * randn(1, length(ofdm_signal)));

r = sqrt((N + Ncp) / N) * ofdm_signal + 10^(-EsN0dB(i) / 20) * noise;

% ----------------- Receiver ----------------------

% Removing cyclic prefix

r_Parallel = r(Ncp + 1:(N + Ncp));

% FFT Block

r_Time = sqrt(Nst) / N * fft(r_Parallel);

% Extracting the data carriers from the FFT output

R_Freq = r_Time([(2:Nst/2+1), (Nst/2+13:Nst+12)]);

% BPSK demodulation / Constellation Demapper

R_Freq(R_Freq > 0) = +1;

R_Freq(R_Freq < 0) = -1;

s_cap = R_Freq;

% Count number of errors

numErrors = sum(abs(s_cap - s) / 2);

% Accumulate bit errors for all symbols transmitted

errors(i) = errors(i) + numErrors;

end

theoreticalBER(i) = (1/2) * erfc(sqrt(10.^(EbN0dB(i) / 10))); % BER for BPSK over AWGN

end

% Compute Simulated BER

simulatedBER = errors / (nSym * Nst);

% Plot results

plot(EbN0dB, log10(simulatedBER), 'r-o');

hold on;

plot(EbN0dB, log10(theoreticalBER), 'k*');

grid on;

title('BER Vs EbN0dB for OFDM with BPSK modulation over AWGN');

xlabel('Eb/N0 (dB)');

ylabel('BER');

legend('Simulated', 'Theoretical');

Output: