Program:

function [meanDelay, rmsDelay, symbolRate, coherenceBW] = meas_continuous_PDP(fun, lowerLim, upperLim)

% Function to calculate mean Delay, RMS delay spread, maximum symbol

% rate that a signal can be transmitted without ISI, and the coherence

% BW for the PDP equation specified as function handle (fun)

% Example: fun = @(tau) exp(-tau/0.00001); % Given PDP equation

% lowerLim - Lower limit for integration

% upperLim - Upper limit for integration

moment_1 = @(x) x .* fun(x);

meanDelay = integral(moment_1, lowerLim, upperLim) / integral(fun, lowerLim, upperLim);

moment_2 = @(y) ((y - meanDelay).^2) .* fun(y);

rmsDelay = sqrt(integral(moment_2, lowerLim, upperLim) / integral(fun, lowerLim, upperLim));

symbolRate = 1 / (10 * rmsDelay); % Maximum symbol rate to avoid ISI

coherenceBW = 1 / (50 * rmsDelay); % For 0.9 correlation

% coherenceBW = 1 / (5 * rmsDelay); % For 0.5 correlation

end

Script:

% Define the Power Delay Profile (PDP) function

fun = @(tau) 2 * exp(-tau / 1e-6);

% Call the function with integration limits

[meanDelay, rmsDelay, symbolRate, coherenceBW] = meas_continuous_PDP(fun, 0, 10e-6);

% Display results

fprintf('Mean Delay: %e seconds\n', meanDelay);

fprintf('RMS Delay: %e seconds\n', rmsDelay);

fprintf('Symbol Rate: %e Hz\n', symbolRate);

fprintf('Coherence Bandwidth: %e Hz\n', coherenceBW);

% Generate PDP curve

tau = 0:0.01e-6:5e-6;

fun1 = 2 * exp(-tau / 1e-6);

% Plot PDP

figure;

plot(tau, fun1, 'r', 'LineWidth', 2);

title('Power vs Delay', 'FontSize', 20);

xlabel('Delay (s)', 'FontSize', 16);

ylabel('Power (dBm)', 'FontSize', 16);

grid on;

Output: